RSM PowerLine F

Repair Manual

RSM PowerLine F - Repair Manual

Contents

1 General Information . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 1 1.1 Identification data . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 1 1.2 Customer data . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 1

1.3 Introductory information . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 2 1.3.1 Explanation of symbols . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 2 1.3.2 Scope of the repair manual . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 2 1.3.3 Contact addresses . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 3

2 Safety . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 7 2.1 Operation according to regulations . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 7 2.2 Operation of the laser . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 8 2.3 General safety instructions . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 8

2.4 Safety notices for the laser system . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 10 2.4.1 Beam path . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 11 2.4.2 Alignment laser . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 12

2.5 Safety instructions for maintenance and setting tasks . . . . . . . . . . . . . . . . . . . . . . . 12 2.6 Electrotechnical safety instructions . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 13 2.7 Electrostatic sensitive devices (ESD) . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 14

2.8 Emergency stop mechanisms and safety equipment . . . . . . . . . . . . . . . . . . . . . . . . 15 2.8.1 Emergency stop mechanisms . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 15 2.8.2 Safety equipment . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 16

2.9 Other dangers . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 17 2.9.1 Ultraviolet radiation . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 17 2.9.2 Hazardous materials . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 17

2.10 Personal protective measures . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 17 2.11 Administrative precautions . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 18 2.12 Waste disposal information . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 18 2.13 Labeling . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 18

3 Required tools, measuring devices, and aids . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 21

4 Commissioning the laser for testing and measuring purposes . . . . . . . . . . . . . . . . . . . . 27 4.1 Settings in the VLM software . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 27 4.2 Settings in the machine configuration . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 29 4.3 Starting the main laser from the VLM software . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 32 4.4 Starting the alignment laser from the VLM software . . . . . . . . . . . . . . . . . . . . . . . . . 33

5 Troubleshooting . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 35 5.1 Image errors . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 35

RSM PowerLine F - Repair Manual iii

5.2 Marking errors . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 40 5.3 Emergency stop circuit . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 41

5.4 Measurement of the laser power . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 42 5.4.1 Measuring points . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 42 5.4.2 Performing the measurement . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 44 5.4.3 Assessment of the measurement . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 46

5.5 Loading a new ALI configuration . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 47 5.6 Checking the ALI system settings . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 50 5.7 Settings in the RCU and VLM software . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 51

6 Cleaning optical components . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 53

6.1 Galvo head . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 54 6.1.1 Protective glass . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 54 6.1.2 Focusing lens . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 55

6.2 Beam expander . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 58 6.3 Laser source . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 59

7 Adjustment tasks . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 61 7.1 Adjusting the laser head . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 61

7.2 Adjusting the laser beam . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 63 7.2.1 Main laser . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 63 7.2.2 Focal point . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 71 7.2.3 Alignment laser . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 73

8 Repair work . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 75

8.1 Changing the optical components . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 75 8.1.1 Galvo head . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 75 8.1.2 Protective glass . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 81 8.1.3 Beam expander . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 82

8.2 Changing the shutter . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 83 8.3 Changing the laser source . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 84 8.4 Changing the PL-CF board . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 101 8.5 Changing the -G1 power supply unit . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 106 8.6 Changing power supplies -G2, -G3 and -G4 . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 109 8.7 Changing the warning lights in the laser head . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 111 8.8 Changing the ALI board . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 112 8.9 Changing the PC . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 115 8.10 Changing the filter pad . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 116

iv RSM PowerLine F - Repair Manual

General Information

1 General Information

1.1 Identification data

Identification data Machine type: Model designation: Serial number: Order number: Parts list number: Manufactured on:

1.2 Customer data

Customer data Inventory number: Location:

RSM PowerLine F - Repair Manual 1

General Information

1.3 Introductory information

1.3.1 Explanation of symbols

1 Attention: This symbol appears in all safety instructions in the repair manual. Possible risks are  thus specially marked.

Non-observance of these safety instructions can lead to severe injury (including death) and/or to  considerable property damage!

3 Note: This symbol indicates information and advice regarding operation and maintenance in the  repair manual.

1.3.2 Scope of the repair manual

1 Attention: This repair manual is intended solely for maintenance personnel trained and  authorized by ROFIN-SINAR for PowerLine E series lasers.

This repair manual is valid for lasers of the ROFIN-SINAR PowerLine E series.

This repair manual must be read, comprehended and observed by the competent maintenance  personnel. ROFIN-SINAR Laser GmbH shall not be liable for damage and operating failure  resulting from the non-observance of the repair manual. The copyright on this repair manual is  exclusively reserved to ROFIN-SINAR. This manual is only entrusted to the owner of the laser for  his/her personal use.

No part of this repair manual nor technical regulations nor drawings may be reproduced, stored in  a retrieval system or transmitted, in any form or by any means, electronic, mechanical,  photocopying, recording, or otherwise without the prior consent of Rofin-Sinar.

3 Note: We reserve the right to make technical changes in the information in this repair manual in  order to improve the laser system.

2 RSM PowerLine F - Repair Manual

General Information

1.3.3 Contact addresses

In case of any questions regarding the content of this repair manual or the laser system itself,  please contact:

ROFIN-SINAR Laser GmbH

Dieselstr. 15

85232 Bergkirchen/Günding

Germany

Phone: +49-8131-704-0

Fax: +49-8131-704-100

E-mail: info@rofin-muc.de

You can also find us at the following locations:

Worldwide locations

ROFIN-SINAR Technologies,  Inc.

40984 Concept Dr.

Plymouth, MI 48170, USA

🕿+1-734-455-5400

+1-734-455 2741

info@rofin.com

ROFIN-SINAR Technologies  Europe S.L.

Plaza Maria Aurella Capmany, 1-A  08970 Sant Joan Despí, Barcelona,  Spain

🕿+34-93 477 4200

+34-93,4774201

ROFIN subsidiaries & production sites

Headquarter Laser Macro ROFIN-SINAR Laser GmbH  Berzeliusstraße 83

22113 Hamburg, Germany

🕿+49-(0)-40-7 33 63-0  +49-(0)-40-7 33 63,160  info@rofin-ham.de

ROFIN-SINAR, Inc.

40984 Concept Dr.

Plymouth, MI 48170, USA

🕿+1-734-455-5400

+1-734-455 2741

info@rofin-inc.com

ROFIN-BAASEL Benelux B.V.  Edisonweg 52

2952 AD Alblasserdam,  Netherlands

🕿+31-(0)-78-6931037

+31-(0)-78-6931079  info@rofin-baasel.nl

Headquarter Laser Micro

Carl Baasel Lasertechnik GmbH & Co.  KG

Petersbrunner Straße 1b

82319 Hamburg, Germany

🕿+49-(0)-8151-776-0

+49-(0)-8151-776 159

sales@baasel.de

ROFIN-BAASEL, Inc.

330 Codman Hill Road

Boxborough, MA 01719, USA

🕿+1-978-635-9100

+1-978-635 9199

info@rofin-baasel.com

ROFIN-BAASEL España, S.L.  Pol. Arazuri-Orcoyen, Calle C, no. 12  31170 Arazuri

Navarra, Spain

🕿+34-948-324-600

+34-948-324 -605

info@rofin-es.com

Headquarter Laser Marking ROFIN-SINAR Laser GmbH  Dieselstraße 15/Günding  85232 Bergkirchen, Germany

🕿+49-(0)-81 31-704-0

+49-(0)-8131-704,100  info@rofin-muc.de

ROFIN-BAASEL Canada Ltd. 3600A Laird Road Unit 15 Mississauga, ON

CANADA L5L 6A6

🕿+(905) 607-0400

+(905) 607-0655

info-canada@rofin-inc.com

ROFIN-BAASEL France S.A.  10, Allée du Cantal

Z.I. La Petite Montagne Sud 91018 Evry Cedex, France

🕿+33-(0)-1-6911-3636  +33-(0)-1-6911-3639 info@rofin.fr

RSM PowerLine F - Repair Manual 3

General Information

ROFIN-BAASEL Italiana S.r.l.  Viale Lombardia, 159

I20052 Monza (MI), Italy

🕿+39-039-2729-1

+39-039-2141304

info@rofin.it

ROFIN-BAASEL China Co., Ltd  Room 206, Bldg. 2

No. 1077 Zu Chongzhi Road 201203 Shanghai

P.R.China

🕿+86-21-68 55 22 16

+86-21-50 27 37 93

info@rofin-baasel.com.cn

ROFIN-BAASEL Singapore Pte.  Ltd.

Block 5012, Ang Mo Kio Av. 5  #04-05 TECHplace II

Singapore 569876

🕿+65 6482-1091

+656482-1158

reception@rofin-baasel.com.sg

ROFIN sales & service

ROFIN-BAASEL Benelux B.V. Brussels Office

Rue Abbé Cuypers, 3

B-1040 Brussels, Belgium

🕿+32-(0)-2-74 12-427

+32-(0)-2-7412,404

ROFIN-SINAR Laser GmbH Switzerland Office

Im Seewinkel 26

3645 Gwatt/Thun, Switzerland

🕿+41-33-3366-690

+41-33-3366 604

ROFIN-SINAR UK Ltd.

York Way, Willerby,

Kingston upon Hull

HU10 6HD, United Kingdom

🕿+44-(0)-1482-6500-88

+44-(0)-1482-650022

info@rofin-uk.com

ROFIN-BAASEL Japan Corp. 1042-4 Toda, Atsugi-shi

Kanagawa-ken, Japan 243-0023

🕿+81-(0)-462-298-655

+81-(0)-462-298-541

info@rofin-jpn.co.jp

ROFIN-BAASEL Taiwan Ltd.  2F, No. 35, Lane 21, Sec 6 Ming  Chuan East Rd.

Taipei, Taiwan

🕿+886-2-2790 1300

+886-2-2795 -3021

info@rofin-baasel.com.tw

ROFIN-BAASEL España, S.L. Barcelona Office

Plaza Maria Aurella Capmany, 1-A  E-08970 Sant Joan Despí, Spain

🕿+34-93-4770644

+34-93-4770865

barcelona@rofin-es.com

ROFIN-BAASEL, Inc.

Tempe Office

1565 W. University Drive

Suite 101

Tempe, AZ 85281, USA

🕿+1-480-777-1199

+1-480-517 9684

ROFIN-BAASEL UK Ltd.

Sopwith Way

Drayton Fields Industrial Estate Daventry NN11 8PB

Northants, United Kingdom

🕿+44 (0) 1327 701100

+44 (0) 1327 701110

sales@rofin-baasel.co.uk

ROFIN-BAASEL Korea Co., Ltd. 602 World Meridian Venture Center 60-24 Gasan-dong Gumchun-gu Seoul, Korea 153-801

🕿+82-2-837-1750

+82-2-837 -1751

info@rofin-baasel.co.kr

ROFIN-BAASEL France S.A.  Sartrouville Office

7, Rue d'Estienne d'Orves

F-78508 Sartrouville Cedex, France

🕿+33-(0)-1-395-77133

+33-(0)-1-395-76577

info-marquage@rofin.fr

ROFIN-BAASEL China Co., Ltd  Shenzhen Office

Room 3684, Floor 3

LongSheng Building, Longhua Bao An District, Shenzhen

P.R. China (518109)

🕿+86-139 252 490 86

4 RSM PowerLine F - Repair Manual

General Information

Further RSTI group companies

DILAS Diodenlaser GmbH  Galileo-Galilei-Straße 10 55129 Mainz, Germany

🕿+49-(0)-6131-9226-0  +49-(0)-6131-9226255  sales@dilas.de

ES Technology Ltd

Units H1 + 2

Kingston Business Park,  Kingston Bagpuize

Oxfordshire, OX13 5SF, UK

🕿+44-(0) 1865-821-818 +44-(0) 1865-821-044 sales@estechnology.net

m2k-laser GmbH

Tullastraße 72

79108 Freiburg, Germany

🕿+49-761-5565-612

+49-761-5573 -332

info@m2k-laser.com

PRC Europe N.V.

Industriepark De Bruwaan 35C - B-9700

Oudenaarde, Belgium

🕿+32-(0)55-30 31 96

+32-(0)55-30 94 96

sales@prc-europe.be

RASANT-ALCOTEC

Beschichtungstechnik GmbH  Zur Kaule 1

51491 Overath, Germany

🕿+49-(0)-2206 9025-0  +49-(0)-2206902522  info@rasant-alcotec.de

DILAS Inc.

9070 South Rita Road

Suite 1500

Tucson, AZ 85747, USA

🕿+1-520 232 3480

+1-520,2323499

sales@dilas-inc.com

H2B Photonics GmbH

An der Universität 2

30823 Garbsen, Germany

🕿+49 (0)511-762-18250

+49 (0)511-762-18252

info@h2b-photonics.de

Optoskand AB

Krokslätts Fabriker 30

S-431 37 Mölndal

🕿+46 (0)31-706 27 50

+46 (0)31-706 27 78

info@optoskand.se

WEGMANN-BAASEL

Laser und elektrooptische Geräte  GmbH

Münchner Str. 15b

85604 Zorneding, Germany

🕿+49-(0)-8106-30626-0

+49-(0)-8106-30626-13  info@wb-laser.de

Corelase Oy

Vesiroineenkatu 3, P.O.Box 73  33721 Tampere, Finland

🕿+358 (0)20-769-9900 +358 (0)20-769-9901  info@corelase.fi

LEE LASER, Inc.

7605 Presidents Drive Orlando Florida 32809, USA

🕿+1-407-812-4611

+1-407-850 -2422

PRC LASER

North Frontage Road

Landing, NJ 07850, USA

🕿+1-973-347-0100

+1-973-347 -8932

sales@prclaser.com

PMB Elektronik GmbH

Leutstettener Str. 28

82319 Hamburg, Germany

🕿+49-(0)-8151 91691-0  +49-(0)-81519169166

RSM PowerLine F - Repair Manual 5

General Information

6 RSM PowerLine F - Repair Manual

Safety

2 Safety

This laser system may only be installed, operated, serviced and repaired by specially trained  personnel who have received instruction concerning the hazards involved in its operation. This  includes reading this repair manual and especially this chapter.

Please contact ROFIN-SINAR Laser or the system manufacturer if you have questions regarding  this chapter or the safety of the laser system in general.

1 Attention: The safety instructions in the other laser system documents (operating instructions,  maintenance manual) and the peripheral component documents must also be observed! These  documents remain valid without limitations!

2.1 Operation according to regulations

The laser system has been designed to mark workpieces. The use of the laser for applications  other than the intended ones are considered misuse and the laser manufacturer is not liable for  any damage thereby caused. In this case, the user assumes the entire responsibility.

The laser manufacturer is not liable for damage caused by modifications made to the machine  without consulting the manufacturer.

ROFIN-SINAR lasers have been manufactured in accordance with the following safety  regulations:

– EN ISO 12100

– EN 60204

– EN 60825

– VDE 0837 (IEC 825)

– UVV BGV B2

– BGI 832

– VDE 0100

– VDE 0105

– 21 CFR - National Center for Devices and Radiological Health - CDRH No. 0121857-004 ; ROFIN-SINAR lasers correspond to the valid EU guidelines:

– 73/23/EEC (Low-Voltage Directive)

– 89/336/EEC (EMC Directive)

– 98/37/EC, Appendix IIA (Machinery Directive [if there is machine status])

RSM PowerLine F - Repair Manual 7

Safety

2.2 Operation of the laser

The rules for the prevention of accidents by laser radiation (UVV) BGV B2 (formerly VBG 93) must  be observed when operating the laser in the area within which these rules and relating instructions  are valid. The implementation of the accident prevention regulation UVV BGV B2 "Laser  Irradiation" can be found in BGI 832 - Operation of Laser Equipment. Under UVV BGV B2, a  person must be appointed to take charge of laser safety, and the professional association and the  appropriate work safety authorities for lasers from class 3B or 4 must also be informed.

Outside the area within which the BGV B2 is valid, the national regulations of the user’s country  must be observed with regard to the rules for prevention of accidents by laser radiation.

The laser system may only be operated by trained and authorized personnel. Training courses are  offered by:

– ROFIN-SINAR Laser GmbH (maintenance, repair application, operation)

– OEM suppliers (operation)

– Professional Association of Precision Mechanics and Electronic Textile Technique (UVV) – PTB Physical and Technical Federal Institute Braunschweig (UVV)

– Technical Supervisory Associations (UVV)

2.3 General safety instructions

• The plant owner must make sure that no unauthorized personnel work on the laser system or  in its vicinity.

• The laser system may be operated only by qualified personnel. The plant owner is  responsible for the selection and training of the personnel. The personnel must have annual  training on laser-specific risks. This training must be documented. The personnel must be  taught to handle the laser system at set intervals, for which a written record must be kept.

• The laser system may be operated only in a malfunction-free state. Safety equipment may be  neither dismantled nor deactivated, not even upon instruction.

The machine's own safety technology should be checked at regular intervals for proper  function and effectiveness.

• In case of changes to the laser system that impair safety, the laser system must be shut  down. Before turning the laser system on again, the faults have to be remedied.

• Due to a possible risk of injury, the laser system should be clearly surveyable and clean.  • The personnel is obliged to wear the required personal protective equipment (PSA). E. g.  protective goggles with the corresponding protective class adapted to the wavelength of the  laser (see BGI 5092 "Selection and use of protective goggles and adjustment goggles"). • Every manner of working that impairs the safety of the laser system or people is prohibited.  Improper use of the laser system must be ruled out.

• Operating the system under the influence of drugs, alcohol or medications that influence  perception and reactions is prohibited!

• When switched on, the laser system must be monitored by operating personnel. Personnel  must refrain from any manner of working that impairs safety.

• The plant owner is obliged to check the system for visible damage and faults at least once a  shift. Changes that appear that impair safety must be eliminated immediately.

• During operation, you absolutely may not reach into the working range of the laser system  with your hand or tools; safety equipment may not be evaded. A risk of injury exists! • The laser system may only be operated using the provided controls. In this case, the use of  tools (screwdrivers, or similar) is prohibited.

8 RSM PowerLine FRepair Manual

Safety

• Adjustment tasks may only take place during the setup mode. Personnel must take special  care. These adjustment tasks may be performed only by qualified personnel using the  provided controls.

• If unexpected risks arise during the operation of the laser system, operation must stop until  the risks have been eliminated.

• The responsibilities for the various tasks within the scope of the operation of the system must  be clearly defined. This applies particularly for work on electrical equipment and on beam guiding components.

• Electrically unsecured doors or covers that can be opened or removed only with tools may be  opened or removed only when the main switch is turned off.

• Before powering up the laser system, all tools and aids must be removed from the working  area to exclude an endangerment of people and property.

• When the laser system is shut down, the main switch should be shut off and secured. • In case of unexpected risk situations, the laser system must be shut down immediately using  the emergency stop button.

• Emergency stop mechanisms may not be used as off switches in normal situations. • After the emergency stop is pressed or after a serious fault, a safety check is required. • Work in the electrical switch cabinet or on the control panel or electrical system may only be

performed by qualified personnel. Control and switch cabinets must always be kept closed. • Wet and compressed-air cleaning of the laser system is prohibited and only permitted under  the following conditions:

– Wet-clean the outside with mild soapy solution or mild cleaning agents.

– Compressed air from a can (water-free and oil-free, nitrogen (1 - 2 bar).

1 Attention: There is a risk of suffocation in case the nitrogen concentration in the ambient air is  too high! Never exceed the permissible limit values!

• Supply lines for the laser system (electrical power, cooling water) are to be laid in cable seats  so that no one can trip over them.

• During work performed on additionally purchased function parts, the technical documents of  the manufacturers must be observed.

• All safety notices and warnings attached to the laser system may not be removed and must  always be legible. Damaged or illegible safety symbols must be exchanged immediately.

RSM PowerLine F - Repair Manual 9

Safety

2.4 Safety notices for the laser system

1 Attention: Laser radiation is very hazardous for the eyes. Radiation of the skin can cause  severe burns. Diffuse, secondary (reflected) radiation is also dangerous. Laser radiation can  cause fire risk or risk of explosion. Avoid any exposure to direct or secondary laser radiation at all  times during installation, operation, maintenance or service of the laser. Never look into the laser  beam directly or with optical devices. The organizational and personal protective measures must  be observed!

1 Attention: Laser radiation can lead to permanent eye damage and skin burns!

1 Attention: After the cover of the laser head has been opened, the laser beam can be accessed  freely! Laser operation with removed cover is prohibited! After completing service and repair work  for which the laser head must be opened, the labels ("Do not open") must be pasted over the two  top fastening screws of the laser head cover again!

1 Attention: Warning against laser radiation!

A laser without any special protective unit corresponds to hazard class 4. A laser beam is invisible  at a wave length of 1064 nm. If the positioning laser is activated (wavelength 660 nm, red laser  beam, hazard class 3R), the laser beam is visible.

Laser class 1

Lasers that are safe under reasonably predictable conditions; this includes the use  of optical instruments for the direct observation of the beam.

The accessible laser radiation is classified as safe.

Laser class 3R

Lasers emitting in the waverange of 302.5 nm to 1,000,000 nm (1,000 µm) and

where directly looking into the beam may be hazardous. The limit of accessible

emission is within five times of the limit value (GZS) of class 2 in the wavelength

range of 400 nm to 700 nm and within five times of the limit value (GZS) of class 1  for other wavelengths.

"Visible laser radiation" is present if the emitted radiation lies within of the visible

radiation (wavelength range between 400 nm and 700 nm).

Laser class 4

Lasers that can also generate dangerous, diffuse reflections. They can lead to skin  injuries and the risk of fire. Their use requires extreme caution.

"Invisible laser radiation" is present if the emitted radiation lies outside of the visible  radiation (wavelength range between 400 nm and 700 nm).

These areas are to be labeled with the corresponding warning symbols and the

additional text "INVISIBLE LASER RADIATION – avoid irradiation of eyes and skin  through direct or scattered radiation – LASER CLASS 4."

1 Attention: Depending on the layout of the laser marker, the beam is emitted from the optics of  the marking head. The optics in these beam guide systems can point in any direction (360°).

1 Attention: If no galvo head is attached, the laser beam is emitted out of the front of the laser  head.

10 RSM PowerLine FRepair Manual

Safety

• The owner of a laser system of laser classes 3B or 4 is obliged to name a laser protection  officer for the operation of the plant.

• When the protective housing is installed completely, the machine has laser protection class 1.  This means that no dangerous radiation can escape from the protective paneling and thus no  risk exists for the machine operator or other people in the vicinity.

1 Attention: If the machine is serviced and the protective paneling is removed, the machine has  laser protection class 4. In this state, all applicable laser protection measures must be complied  with.

1 Attention: The protective goggles used must be adapted to the wavelength of the laser and  have the corresponding protection class.

• To counteract potential malfunctions arising from inadvertent laser radiation, we remind you to  comply with the proper use of the safety shutter (hereafter "Shutter").

1 Attention: If the laser system has to be switched on for testing/measuring purposes, measures  for protection against laser radiation must be taken (use of protective goggles, setup and  positioning of partition walls, attachment of warning signs and barriers, etc.). These measures  must be coordinated with the laser protection officer.

The DIN EN 207 Filter und Augenschutzgeräte gegen Laserstrahlung (BS EN 207 Filters and Eye Protectors Against Laser Radiation (Laser Eye-Protectors)), DIN EN 60825-1 Sicherheit von  Lasereinrichtungen (BS EN 60825-1 Safety of Laser Products), and DIN EN 60825-4 Sicherheit  von Laserschutzwänden (BS EN 60825- 4 Safety of Laser Products: Laser Guards) standards  must be observed.

2.4.1 Beam path

Figure 2.1 Beam path

The dashed line designates the beam path of the laser. The same applies to to the alignment laser  (if applicable).

RSM PowerLine F - Repair Manual 11

Safety

2.4.2 Alignment laser1

The alignment laser is a laser diode of protection class 3R. The accessible laser emission is within  the wavelength range from 302.5 nm to 106 nm and is hazardous for eyes. The power or energy  equals a maximum of five times of the limit value of the accessible radiation of class 2 in the  wavelength range of 400 nm to 700 nm and within five times of the limit value of class 1 for other  wavelengths.

Remark:

Class 3R laser devices are potentially hazardous to eyes as are Class 3B laser devices. The risk  of eye injury is lower because the limit value of accessible radiation (GZS) in the visible  wavelength range is limited to five times of the limit value of the accessible radiation (GZS) of class  2, and in the other wavelength ranges to five times of the limit value for accessible radiation (GZS)  of class 1.

For continuously radiating lasers of class 3R, the limit value of accessible radiation (GZS) P limit =  5 mW (with small sources C6 = 1) in the wavelength range 400 nm to 700 nm.

1 Attention: Never look into the beam of the alignment laser directly or with optical devices.

2.5 Safety instructions for maintenance and setting tasks

1 Attention: During the performance of maintenance, repair, setting, and monitoring tasks, the  system must be shut down and secured against restarting. The warning sign "System shut down  - activation prohibited!" must be attached to the laser system.

1 Attention: For the observance of laser protection class 1, the operation of the system with  partially or completely dismantled protective housing is fundamentally prohibited.

• After the mounting of the electrical systems or servicing, the function of the existing protective  devices should be checked and the protective measures tested.

• Electrically unsecured doors or covers that can be opened or removed only with tools may be  opened or removed only when the main switch is turned off.

• Work in the electrical switch cabinet or on the control panel or electrical system of the  machine may only be performed by qualified personnel. Control and switch cabinets must  always be kept closed.

• If safety devices are removed during repair work, the machine may not be restarted until all  safety devices have been attached and checked for proper function.

• During maintenance work, make sure that electrical and media lines are not damaged or  crushed.

During this work, the main switch should be shut off and secured.

1 Attention: Laser radiation danger: If parts of the protective housing are removed for work on  beam-guiding units, operation of neighboring systems must be shut down. Authorized personnel  located in the vicinity of the laser system during this work must wear protective goggles according  to DIN 207. The working area must be labeled (laser class 4) (see Section 2.4, page 10).

1. Installation depends on the respective laser type

12 RSM PowerLine FRepair Manual

Safety

1 Attention: After the cover of the laser head has been opened, the laser beam can be accessed  freely! Laser operation with removed cover is prohibited! After completing service and repair work  for which the laser head must be opened, the labels ("Do not open") must be pasted over the two  top fastening screws of the laser head cover again!

1 Attention: The mechanical processing of parts of the laser system (grinding, drilling, separating,  etc.) must fundamentally take place outside of the working area! The processing of support  subassemblies is fundamentally prohibited!

2.6 Electrotechnical safety instructions

• Wiring, electrical connection, commissioning, maintenance, and repair may only be  performed by qualified electricians.

• Work may not be performed on live parts under any circumstances. The system or parts of it  must be electrically disabled and secured against unintentional reactivation.

• When working on the electrical systems, turn off and secure the main switch.

1 Attention: High voltage! Extreme danger!

The devices marked with lighting bolts signs in the switch cabinet (main switch, repair socket,  mains connection terminals) remain live after the main switch is turned off.

The switch cabinet may only be opened by electricians for repair purposes.

1 Attention: Additional devices connected to the laser system may have their own mains supply  line and thus be live when the main switch of the system is turned off.

• The control and main power lines are to be laid separately from each other.

Non-observance may result in the following:

– Failure of the machine function

– Endangering malfunctions

– Destruction of electrical and mechanical components

• Electrical equipment must be checked regularly. Loose connections must be retightened.  Damaged lines or cables should be exchanged immediately.

• The switch cabinet and all electrical supply units must always be kept locked. Access is  permitted only to authorized personnel with a key or special tool.

• PCBs or plug connection may be removed only when the system is shut down. Do not mix up  PCBs or plug connections. The label or coding must be observed.

• In case of measurements on live subassemblies or lines, a second person must always be  present who can turn off the main switch in case of an emergency.

• When working with grounded measuring devices (oscilloscope), make sure that the ground  bushing of the measuring device is always connected with the ground point of the control  system (measuring cable). According to requirements, isolation amplifiers should be used for  perfect measurements.

1 Attention: Never clean electrical equipment with water or other liquids.

RSM PowerLine F - Repair Manual 13

Safety

2.7 Electrostatic sensitive devices (ESD)

1 Attention: The electronic components of the PC plug-in are electrostatic sensitive devices  (ESD)! Protective measures must be observed when working on these components!

Protective measures to be performed:

• Ensure constant equipotential bonding!

• Make sure that personnel is grounded with wristbands and shoes!

• Make sure that clothing is closed and has discharge capacity!

• Materials that can charge electrostatic energy such as normal PE, PVC, styrofoam, etc. must  be avoided!

• Avoid electrostatic fields >100 V/cm!

• Use only marked and defined packaging and transport materials!

1 Attention: When the power connection is disconnected as required prior to opening the  PC plug-in, the PE connection will also be disconnected!

The PC plug-in must be properly grounded before work is performed on electronic  components! The external connection to ground must be established at a marked PE  terminal of the plug-in and a marked PE terminal in the system area! A measurement must  be performed to ensure correct equipotential bonding). The ESD wrist band must then be  connected to a PE terminal of the plug-in!

No work may be performed on electronic components prior to that!

1 Attention: All personnel working on electronic components must comply with the required  measures to protect electrostatic sensitive devices! The work area must be secured!

Figure 2.2 PC plug-in

14 RSM PowerLine FRepair Manual

Safety

2.8 Emergency stop mechanisms and safety equipment

2.8.1 Emergency stop mechanisms

The emergency stop is caused by operating the red emergency stop push button. The emergency  stop switches off the laser as quickly as possible and stops all movements of the operation  process and used to prevent injury to persons, or damage to the machine or workpiece.

Depending on the system layout, the Emergency stop pushbutton is located on the external  control panel or the supply plug-in.

Figure 2.3 Emergency stop push button external control panel

Figure 2.4 Emergency stop push button supply plug-in

RSM PowerLine F - Repair Manual 15

Safety

3 Note: The emergency stop should not be used to end a normal working cycle.

2.8.2 Safety equipment

Figure 2.5 Warning lamp laser head

There are two warning lamps opposite each other in the laser head.

These warning lamps will illuminate when the laser system is switched on with the main switch  and the key switch is set to position I (system closed in operating mode/shutter) or position II  (Open activation system in operating mode/shutter, i. e. when the laser radiation is generated.

The warning lamps flash when an error occurs.

16 RSM PowerLine FRepair Manual

Safety

2.9 Other dangers

2.9.1 Ultraviolet radiation

1 Attention: The metal vapor plasma which emerges when welding and marking certain metals  with a laser beam emits intense invisible ultraviolet radiation which can cause severe damage to  the eyes and the skin!

1 Attention: The laser protective goggles for 1064 nm or 532 nm specified by DIN EN 207 or EN  208 do not offer protection from this ultraviolet radiation.

3 Note: Special goggles which protect the eyes against ultraviolet radiation are commercially  available.

2.9.2 Hazardous materials

1 Attention: Hazardous fumes and dust may be given off during the marking process or when  carrying out service tasks (e. g. replacing contaminated filters). The regulations of the supplier in  regard to safety must be observed. When handling hazardous materials, a suitable breathing  mask and protective gloves shall be used.

2.10 Personal protective measures

1 Attention: Laser radiation is very hazardous for the eyes. Radiation of the skin can cause  severe burns. Diffuse, secondary (reflected) radiation is also dangerous. Laser radiation can  cause fire risk or risk of explosion.

1 Attention: After the cover of the laser head has been opened, the laser beam can be accessed  freely! Laser operation with removed cover is prohibited! After completing service and repair work  for which the laser head must be opened, the labels ("Do not open") must be pasted over the two  top fastening screws of the laser head cover again!

• Avoid any exposure to direct or secondary laser radiation at all times during installation,  operation, maintenance or service of the laser. Never look into the laser beam directly or with  optical devices. The organizational and personal protective measures must be observed!

• Wear suitable protective goggles in the laser area according to DIN EN 207 and EN 208 – eye  protection against laser radiation at a wavelength of 1064 (660) nm.

RSM PowerLine F - Repair Manual 17

Safety

2.11 Administrative precautions

The following regulations must be observed. The detailed organizational protective measures and  guidelines of DIN EN 60825, Classification VDE 0837 (IEC 825), must be observed.  • The operators must be instructed in safety regularly.

• Attach warning signs to warn against laser radiation.

• Deny access to the laser system to anybody not working on it.

• The laser area must be sufficiently labeled.

• Due to a possible risk of fire and explosion, no flammable or easily flammable gases, liquids,  or solids may be brought into the laser area.

• Toxic decomposition products may develop when certain materials (e. g. metals or plastics)  are processed. Information on possible risks that can appear should be obtained, e. g. from  professional organizations.

• Objects that can endanger people through the uncontrolled reflection of the laser radiation  must be removed from the laser area.

• Have authorized and purposely appointed persons check the effectiveness of integrated  safety equipment (e. g. emergency stop) according to defined test cycles. All relevant national  safety regulations and guidelines must be observed (see "Operation according to regulations"  on page 7).

Existing safety equipment must be actuated during operation. Any endangering functions  must immediately be stopped or interrupted. Before renewed powering up the laser system,  the corresponding displays or error messages must be acknowledged. Make sure that the  laser system can be restarted. If this is confirmed the safety equipment is in proper working  order.

• Make sure that the warning lamps regarding laser radiation function properly. A defective  lamp must be replaced immediately. The laser radiation warning lamps are located on top of  the laser head.

2.12 Waste disposal information

Comply with all national and regional regulations regarding waste disposal.

2.13 Labeling

All locations which, under certain circumstances (e. g. when protective covers are opened),  represent a potential risk, are labeled with the required warning labels. The location of the  individual signs are marked in the Safety chapter in the user manual.

1 Attention: These labels must not be removed.

18 RSM PowerLine FRepair Manual

Safety

Notes

RSM PowerLine F - Repair Manual 19

Safety

20 RSM PowerLine FRepair Manual

Required tools, measuring devices, and aids

3 Required tools, measuring devices, and aids

In addition to a standard tool set, the tools, measuring devices, and aids listed in the following are  required for performing repair work on lasers of the RSM PowerLine F series:

• Protective goggles*

1 Attention: The protective

goggles must be adapted to the

wavelength of the laser that is used.

3 Note: The protective goggles

shown in Figure 3.1 are suitable for all

wavelengths that may occur with the RSM

PowerLine F. If protective goggles are

used that do not cover the entire

wavelength range, multiple protective

goggles must be used accordingly.

Figure 3.1 Protective goggles

• Power measurement device including

sensor and fitting for power

measurement head LM 200*

1 Attention: When the power

measurement device is used, protective

goggles adapted to the wavelength of the

laser must be used!

Figure 3.2 Power measurement device

• Adjustment tube of galvo head with

base plate*

Figure 3.3 Adjustment tube of galvo head with

base plate

RSM PowerLine F - Repair Manual 21

Required tools, measuring devices, and aids

Figure 3.4 Cross wires with seat

Figure 3.5 Transducer disk

Figure 3.6 Multimeter

• Cross wires with seat D=16 mm, cross  wires D=25 mm*

• Transducer disk "Beam catcher"* 1 Attention: When the transducer  disk is used, protective goggles adapted  to the wavelength of the laser must be  used!

• Multimeter

22 RSM PowerLine F - Repair Manual

Required tools, measuring devices, and aids

Figure 3.7 Fine measuring tips

Figure 3.8 Measuring adapter for galvo voltage

Figure 3.9 Scotch tape no. 6877 50x66

• Fine measuring tips

3 Note: Various measuring points  cannot be reached with standard  measuring tips.

• Measuring adapter for galvo voltage*

• Scotch tape no. 6877 50x66* 1 Attention: Only tape that can be  completely removed without residue may  be used!

RSM PowerLine F - Repair Manual 23

Required tools, measuring devices, and aids

Figure 3.10 Precision level gauge

Figure 3.11 Lens cleaning paper pack

Figure 3.12 Isopropyl alcohol

• Precision level gauge

• Lens cleaning paper pack*

• Isopropyl alcohol for the cleaning of  optical components*

24 RSM PowerLine F - Repair Manual

Required tools, measuring devices, and aids

Figure 3.13 Compressed air can

Figure 3.14 Rubber gloves and dust protection mask

Figure 3.15 ESD wrist band with spiral cord

• Compressed air can*

• Rubber gloves and dust protection  mask for cleaning tasks*

• ESD wrist band with spiral cord*

RSM PowerLine F - Repair Manual 25

Required tools, measuring devices, and aids

Figure 3.16 M-Function PL-CF test box

• M-Function PL-CF test box* The test box can be used to externally  activate the functions of the laser. The  connection is established at plugs -X42a  and -X42b of the supply plug-in.

*. For the order number, see the ROFIN-SINAR measuring and adjusting device catalog

26 RSM PowerLine F - Repair Manual

Commissioning the laser for testing and measuring purposes

4 Commissioning the laser for testing and  measuring purposes

4.1 Settings in the VLM software

3 Note: The main or alignment laser will only start while a VLM program is running.

1. Starting the PC.

2. Starting the VLM software.

3. Create the ellipse and assign a

height and width of 2 mm in the

properties.

Figure 4.1 Ellipse properties - dimensions

4. Calling "Drawing settings" under

"Options".

Figure 4.2 Calling drawing settings

5. Check the "Multiple marking" box.

6. Click the [Apply] and [OK] buttons to

activate changes.

Figure 4.3 Drawing settings

RSM PowerLine F - Repair Manual 27

Commissioning the laser for testing and measuring purposes

Figure 4.4 Ellipse properties - LP type

1 2 Figure 4.5 Start laser

7. Call "Ellipse properties" again and  select the "LP type" tab.

8. Enter the values for current and  frequency.

3 Note: Do not enter an absolute  value under "Cur.(A)" but a percentage of  the maximum current.

9. Copy the entry with the "Add" button  until a total of about 50 to 60 repeats  are performed and set the speed [V]  to 10 mm/s.

3 Note: The laser is only in  operation during marking.

10. Start the program and thereby the  main laser using Button (1)

(Figure 4.5) or the alignment laser  using Button (2).

3 Note: The main or alignment  laser can only be started individually. The  two lasers cannot be operated

simultaneously.

28 RSM PowerLine F - Repair Manual

Commissioning the laser for testing and measuring purposes

4.2 Settings in the machine configuration

3 Note: In order to be able to start the laser with disassembled galvo head, enter the following  settings in the machine configuration. These settings must be reset after testing and measuring  has been completed.

1. Starting the PC.

2. Starting the machine configuration.

Figure 4.6 Starting the machine configuration

3. Select the "Galvo 1" (1) tab and click

on the "Expanded" (2) button.

1

2

Figure 4.7 Machine configuration

RSM PowerLine F - Repair Manual 29

Commissioning the laser for testing and measuring purposes

Figure 4.8 Advanced galvo head settings

1 2 Figure 4.9 Activating the machine configuration

4. Uncheck the "Activate galvo check"  box.

5. Click the "OK" button.

6. Click on the "Save" (1) and "Convert  to active machine configuration file"  (2) buttons.

30 RSM PowerLine F - Repair Manual

Commissioning the laser for testing and measuring purposes

Figure 4.10 Enable galvo check

7. Reactivate the galvo check after  completing tests and

measurements.

RSM PowerLine F - Repair Manual 31

Commissioning the laser for testing and measuring purposes

4.3 Starting the main laser from the VLM software

1

Figure 4.11 Start the main laser

The laser must be started according to the instructions in the user manual.

The main laser is started in the VLM software using the corresponding button (1) (Figure 4.11).

3 Note: The main laser can only be started when the alignment laser is out of commission. A VLM  program must be running (see Section 4.1, page 27).

32 RSM PowerLine F - Repair Manual

Commissioning the laser for testing and measuring purposes

4.4 Starting the alignment laser1 from the VLM software 1

Figure 4.12 Starting the alignment laser

The laser must be started according to the instructions in the user manual.

The main laser is started in the VLM software using the corresponding button (1) (Figure 4.11).

3 Note: The main laser can only be started when the alignment laser is out of commission. A VLM  program must be running (see Section 4.1, page 27).

1. Installation depends on the respective laser type

RSM PowerLine F - Repair Manual 33

Commissioning the laser for testing and measuring purposes

Notes

34 RSM PowerLine F - Repair Manual

Troubleshooting

5 Troubleshooting

5.1 Image errors

3 Note: For the precise assessment of the image faults, a measuring magnifier or microscope is  required.

Figure 5.1 Barrel or pillow shaped distortion

| Error Barrel and/or pillow shaped distortions in the X and Y directions | Remedy Load the compensation file belonging to the optics. |
| --- | --- |

Figure 5.2 Compensation file loaded

RSM PowerLine F - Repair Manual 35

Troubleshooting

Figure 5.3 Trapezoidal distortion

| Error Trapezoidal distortions in the X and Y directions | Remedy Check the adjustment of the workpiece support plate and the galvo head (see Section 7.1, page 61). |
| --- | --- |

Figure 5.4 Focusing errors

| Error Focusing error (center point okay, deviation at edge) | Remedy Checking the focal distance of the galvo head (see Section 7.1, page 61). |
| --- | --- |

36 RSM PowerLine F - Repair Manual

Troubleshooting

Figure 5.5 Correct image

3 Note: A test program should be created to test the image (see the VLM user manual).

Sample settings for the test program:

• Square 120 mm x 120 mm (congruent with the maximum marking field of the laser) • Hatching 45°

• Hatching distance approximately 0.5 mm ... 2.0 mm

• The speed, current and frequency must be adapted to the material being marked. The image of the lines must be clean and have a high acutance.

1 Attention: The laser system and the workpiece support plate must be absolutely vibration-free!  The suction system must be switched on since vapors obstruct/weaken the laser radiation!

3 Note: To prevent errors due to the material being marked, let the test program run through  several test patterns.

3 Note: For the precise assessment of the image faults, a measuring magnifier or microscope is  required.

RSM PowerLine F - Repair Manual 37

Troubleshooting

Figure 5.6 Irregular power fluctuations

| Error Irregular power fluctuations | Remedy Check the optics for soiling and burning (see Chapter 6). |
| --- | --- |

Figure 5.7 Regular power fluctuations

| Error Regular power fluctuations due to internal or external influences | Remedy Check or change the laser unit. Check system for vibrations. |
| --- | --- |

38 RSM PowerLine F - Repair Manual

Troubleshooting

2

1

Figure 5.8 Positioning error of galvo head

| Error Positioning error of galvo head (1) (X or Y direction) | Remedy Check/exchange the galvo head or ALI board. Check the system for vibrations. |
| --- | --- |

3 Note: If the test matrix is not hatched in a 45° angle, errors will possibly not be recognized (2).

RSM PowerLine F - Repair Manual 39

5.2 Marking errors

Figure 5.9 Marking errors

Troubleshooting

1

2

3

4

5

6

| No. 1 2 3 4 5 6 | Error No error Distortions The first pulse is too strongly attenuated.LASER-OFF-DELAY too long LASER-OFF-DELAY too short The first pulse is not attenuated enough. | Remedy – • Check the parameters and hardware. • Check the parameters and hardware. • Check the parameters and hardware. • Check the parameters and hardware. • Check the parameters and hardware. |
| --- | --- | --- |

40 RSM PowerLine F - Repair Manual

Troubleshooting

5.3 Emergency stop circuit

1 Attention: The peripheral components of the laser system should be shut down and secured  against reactivation.

• Emergency stop circuit OK  Figure 5.10 Emergency stop relay – OK

• Emergency stop circuit triggered Figure 5.11 Emergency stop relay – Triggered

RSM PowerLine F - Repair Manual 41

Troubleshooting

5.4 Measurement of the laser power

1 Attention: For the measurement of the laser power, a power measurement device with a  measuring adapter for the respective laser type is required (see Chapter 3). The operating  instructions of the measuring device absolutely must be observed!

1 Attention: If the laser system has to be switched on for testing/measuring purposes, measures  for protection against laser radiation must be taken (use of protective goggles, setup and  positioning of partition walls, attachment of warning signs and barriers, etc.). These measures  must be coordinated with the laser protection officer.

The DIN EN 207 Filter und Augenschutzgeräte gegen Laserstrahlung (BS EN 207 Filters and Eye Protectors Against Laser Radiation (Laser Eye-Protectors)), DIN EN 60825-1 Sicherheit von  Lasereinrichtungen (BS EN 60825-1 Safety of Laser Products), and DIN EN 60825-4 Sicherheit  von Laserschutzwänden (BS EN 60825- 4 Safety of Laser Products: Laser Guards) standards  must be observed.

1 Attention: During the measurement of the laser power, a second person must always be  present to press the emergency stop button/main switch of the laser system in case of an  emergency.

1 Attention: The peripheral components of the laser system should be shut down and secured  against reactivation.

5.4.1 Measuring points

m

m

0

0

6

7

.

x

o

t

am

Measuring

p

u

Figure 5.12 Laser power after the galvo head

1 Attention: The laser power after the galvo head must fundamentally be measured outside of the  focus to prevent burning or other damage to the measurement head! At a burning width of e. g.  160 mm, measures a maximum of 60 to 70 mm underneath the protective glass!

42 RSM PowerLine F - Repair Manual

Troubleshooting

1  Figure 5.13 Laser power measuring point on the galvo flange

| No. 1 | Measuring point Galvo flange* |
| --- | --- |

*. Perform the measurement with and without the installed beam expansion and with and without built-in shutter.

| Laser type PWL 10 F PWL 20 F | Power • 60 kHz, 100 % Current: > 9.00 W • 60 kHz, 100 % Current: > 18.00 W |
| --- | --- |

RSM PowerLine F - Repair Manual 43

Troubleshooting

5.4.2 Performing the measurement

1 Attention: Before powering up the laser for measuring purposes, the measurement head  should be positioned at the measuring point!

1 Attention: Do no move the measurement head through the activated laser beam! There is a risk  of an uncontrolled deflection of the laser beam!

1. Secure the shutter against

activation.

2. Disassemble galvo head and enter

settings in the machine configuration

(see Section 4.2, page 29).

1 Attention: Take measures for

protection against laser radiation

(see page 42)!

3. Position the measurement head.

1 Attention: Set the measuring

device to the wavelength of the laser!

Observe the operating instructions of the

Figure 5.14 Measuring the laser power

measuring device!

4. Activate the main switch of the laser  system.

1 Attention: If the emergency stop  button/main switch of the laser system is  out of reach, a second person must  always be present who can press the  emergency stop button/main switch in  case of an emergency.

5. Starting the VLM program (see  Section 4.1, page 27).

44 RSM PowerLine F - Repair Manual

Troubleshooting

Figure 5.15 Key switch / shutter

6. Open the shutter.

7. Perform the measurement. 3 Note: Compare the measured  power with the power after the galvo head  to detect any power losses. The  measurement should be performed with  and without an installed beam expansion  and with and without a built-in shutter to  detect any power losses due to these  components.

1 Attention: Perform the  measurement only for a short time to  prevent damage to the measuring head!  Follow the instructions in the operating  instructions of the measuring device!

8. Close the shutter.

9. Remove the measurement head. 10. Switch off the laser equipment. 11. Completely mount the components. 12. Commission the laser system.

RSM PowerLine F - Repair Manual 45

Troubleshooting

5.4.3 Assessment of the measurement

| Error Power okay, marking result faulty | Possible Reasons Focal distance incorrectly adjusted | Remedy • Adjusting |
| --- | --- | --- |
|  | Focal point incorrectly adjusted | • Adjusting |
|  | Optical components have changed their properties | • Perform further power measurements |
|  | Handling system faulty | • Check it. • Adjusting |
| Power too low | Voltage and current supply | • Checking the power supply |
|  | Laser beam | • Check the beam adjustment. • Check the optical components. |
| Power fluctuations | Voltage and current supply | • Checking the power supply |
|  | Vibrations | • Check it. |
| Power loss | Soiling, protective glass damage, focusing lens, beam expansion, galvo mirror | • Clean, exchange 3 Note: In case of damage to the galvo head, the complete head must be exchanged. |
|  | Galvo head voltage supply fault | • Measure the voltage and adjust it if necessary. • Checking the power supply |
|  | Defective galvo head | • Check its function. • Exchange the galvo head or the ALI card for the optics if necessary. |
|  | Contamination of beam expander: | • Clean it. |
|  | Defective shutter module | • Exchange the shutter module |

46 RSM PowerLine F - Repair Manual

Troubleshooting

5.5 Loading a new ALI configuration  Figure 5.16 VLM PC configuration

Figure 5.17 Selecting the ALI loader file

3 Note: The "New Hardware  Found" dialog is called if the version  difference between the old and new ALI  card is too great (e. g. Version 1 to Version  3).

1. After the change of the ALI card on  the laser PC, call the VLM PC

configuration (VisualLaserMarker >  Configuration > PC Configuration). 2. Call the ALI loader.

3. Press the "FlashFile" button

4. Select and open the corresponding  ALI loader file.

RSM PowerLine F - Repair Manual 47

Troubleshooting

Figure 5.18 Installation ended

Figure 5.19 Laser interface

5. Wait until the installation of the  FlashFile is complete.

3 Note: In this way, any test  program on the ALI card is overwritten.

6. Adapt the settings in the laser  interface.

48 RSM PowerLine F - Repair Manual

Troubleshooting

Figure 5.20 Laser communication

7. Adapt the settings in the laser  communication.

RSM PowerLine F - Repair Manual 49

Troubleshooting

5.6 Checking the ALI system settings  Figure 5.21 Call the computer administration

Figure 5.22 Device Manager

Figure 5.23 Device properties

1. Call the computer administration  (right click on My Computer >  Administer).

2. Open the Device Manager. 3. Under "Multifunction Adapters",  check whether the ALI controller  exists.

4. Check the device properties.

50 RSM PowerLine F - Repair Manual

Troubleshooting

5.7 Settings in the RCU and VLM software

For more information about the installation of the software and required settings, refer to the  applicable software manuals.

RSM PowerLine F - Repair Manual 51

Troubleshooting

Notes

52 RSM PowerLine F - Repair Manual

Cleaning optical components

6 Cleaning optical components

3 Note: The surfaces of the optical components are to be cleaned according to the maintenance  schedule.

3 Note: The ambient conditions must be as dust-free as possible while cleaning optical  components to prevent repeated contamination.

1 Attention: For cleaning, lens cleaning paper and isopropyl alcohol or purified compressed air  should be used!

Cleaning of the optical components:

1. Dismantle optical components as described below.

2. Blow fine dust particles off the surfaces using purified compressed air.

3. Clean optical components using lens cleaning paper and isopropyl alcohol.

– With a pipette, apply 1-2 drops of isopropyl to the surface to be cleaned.

– Apply lens cleaning paper and carefully remove or wipe off the isopropyl alcohol.

1 Attention: Always work with clean, non-greasy hands so as not to soil the optical components!  Use gloves if necessary!

– Repeat the cleaning procedure until the surface of the optics is clean.

1 Attention: Do not damage optical components during cleaning! Do not apply pressure! Remove  the lens cleaning paper only in one direction. Use a new piece of lens cleaning paper each time  you clean!

– Always clean both sides of lenses or partially two-way mirrors. In the process, make sure  that both previously cleaned surfaces are not soiled further.

RSM PowerLine F - Repair Manual 53

Cleaning optical components

6.1 Galvo head

6.1.1 Protective glass

1. Depending on the galvo type, pull the protective glass mount from the protective glass slide  or unscrew the retaining ring of the protective glass.

2. Loosen the respective fastening of the protective glass and remove it.

3. Clean it.

4. Put the protective glass back in place and fasten it.

5. Push the protective glass mount into the protective glass slide or screw the retaining ring of  the protective glass in.

3 Note: Make sure that the protective glass and protective glass mount are installed on the right  side.

1

Figure 6.1 Protective glass

3 Note: The gold colored ring (1) (Figure 6.1) on the protective glass must always be positioned  toward the laser beam (inside of the galvo head). Incorrect installation may lead to marking errors.

54 RSM PowerLine F - Repair Manual

Cleaning optical components

6.1.2 Focusing lens

1 Attention: Switch off the laser system and secure it against reactivation. Pull the mains plug.  The peripheral components of the laser system should also be shut down and secured against  reactivation.

1. Loosen and remove the connecting

1  Figure 6.2 Galvo head connecting plug

1 Figure 6.3 Dismantling the galvo head

Figure 6.4 Covering the entry openings

plug of the galvo head (1)

(Figure 6.2).

2. Loosen the fastening screws (1)  (Figure 6.3).

3. Remove the galvo head.

4. Cover the entry openings with tape  (tape to be used: see Chapter 3) to  prevent the inside of the galvo head  from getting dirty.

RSM PowerLine F - Repair Manual 55

Cleaning optical components

1  Figure 6.5 Removing the protective glass (protective glass slide)

1 Figure 6.6 Remove the protective glass (screwed protective glass)

Figure 6.7 Dismantling the guide plate (protective glass slide)

5. Protective glass slide:Remove the  protective glass (1) (Figure 6.5). Remove screwed protective

glass: Unscrew retaining ring (1)  (Figure 6.6) and remove protective  glass.

3 Note: If the protective glass is  damaged, it must be exchanged (see  Section 8.1.2, page 81).

3 Note: All parts must be marked  for reinstallation with suitable tools.

6. Clean the protective glass  (see page 53).

7. Protective glass slide: Loosen the  screws (Figure 6.7) of the guide  plate for the protective glass.

56 RSM PowerLine F - Repair Manual

Cleaning optical components

1  Figure 6.8 Removing the focusing optics

1 Figure 6.9 Removing the distance ring

Figure 6.10 Checking the cleaning state

8. Unscrew the focusing optics (1)  (Figure 6.8).

9. Clean it (see page 53).

10. Removing the distance ring (1)  (Figure 6.9).

11. Clean the deflecting mirror  (Figure 6.10) (see page 53).

12. Clean (see Section 6.1, page 54) if  necessary.

13. Mount the galvo head in the reverse  order.

3 Note: When mounting the galvo  head, pay attention to the correct position  of the distance ring (see Figure 6.9) and

the position of the protective glass (see  Section 6.1.1, page 54).

14. Remount the galvo head to the laser  and connect it.

15. Commission the laser system.

RSM PowerLine F - Repair Manual 57

Cleaning optical components

6.2 Beam expander1

1 Attention: Switch off the laser system and secure it against reactivation. Pull the mains plug.  The peripheral components of the laser system should also be shut down and secured against  reactivation.

1. Remove the shutter (see

Section 8.2, page 83).

1  Figure 6.11 Loosening the beam expander

mount

1 2 Figure 6.12 Remove the beam expander

1. Installation depends on the respective laser type

2. Loosen the three attachment screws  (1) (Figure 6.11) on the beam

expander mount.

3. Carefully remove the beam  expander (1) (Figure 6.12) and the  mount (2).

3 Note: The current setting of the  beam expander should be noted so that  the correct focal distance can be reset  after any false adjustment of the beam  expander.

4. Unscrew the beam expander from  the mount.

5. Clean it (see page 53).

6. Screw the beam expander into the  mount and reinstall the mount.

1 Attention: Do not crush or jam  the cables and lines inside the laser head  during installation!

7. Check the adjustment of the laser  beam and adjust it if necessary (see  Section 7.2.1, page 63).

8. Check the adjustment of the beam  expander; set the value read before  the cleaning procedure if necessary.

9. Mount the cover of the laser head. 10. Commission the laser system. 11. Check the laser power behind the

galvo head (on the workpiece) using  the power measurement device and  perform a test marking procedure.  Enter the measurement values in  the logbook.

58 RSM PowerLine F - Repair Manual

Cleaning optical components

6.3 Laser source

1 Attention: No cleaning tasks are required on and in the laser source. If necessary, the laser  source should be exchanged as a complete unit. Repair and cleaning inside the laser source may  be performed only by ROFIN-SINAR. In case of non-observance, the warranty is no longer valid.

RSM PowerLine F - Repair Manual 59

Cleaning optical components

Notes

60 RSM PowerLine F - Repair Manual

Adjustment tasks

7 Adjustment tasks

7.1 Adjusting the laser head

1 Attention: Switch off the laser system and secure it against reactivation. Pull the mains plug.  The peripheral components of the laser system should also be shut down and secured against  reactivation.

1 Attention: If the laser system has to be switched on for testing/measuring purposes, measures  for protection against laser radiation must be taken (use of protective goggles, setup and  positioning of partition walls, attachment of warning signs and barriers, etc.). These measures  must be coordinated with the laser protection officer.

The DIN EN 207 Filter und Augenschutzgeräte gegen Laserstrahlung (BS EN 207 Filters and Eye Protectors Against Laser Radiation (Laser Eye-Protectors)), DIN EN 60825-1 Sicherheit von  Lasereinrichtungen (BS EN 60825-1 Safety of Laser Products), and DIN EN 60825-4 Sicherheit  von Laserschutzwänden (BS EN 60825- 4 Safety of Laser Products: Laser Guards) standards  must be observed.

1. Check the X and Y directions of the

position of the workpiece support

plate using a level gauge.

2. Perform an adjustment if necessary.

Figure 7.1 Checking the position of the

workpiece support plate

RSM PowerLine F - Repair Manual 61

Adjustment tasks

Figure 7.2 Checking the position of the galvo head

Figure 7.3 Adjusting the laser head

Figure 7.4 Adjusting the focal distance

3. Check the X and Y directions of the  position of the galvo head using a  level gauge.

4. Adjust the position of the galvo head  using the three-point support plate of  the laser head parallel to the

workpiece support if necessary.

3 Note: If the workpiece support  plate cannot be aligned to be absolutely  straight, the galvo head must be adjusted  in parallel at the value measured under 1.

5. Check the focal distance using the  stickers on the galvo head and the  logbook entries and adjust it if

necessary.

62 RSM PowerLine F - Repair Manual

Adjustment tasks

7.2 Adjusting the laser beam

7.2.1 Main laser

1 Attention: Switch off the laser system and secure it against reactivation. Pull the mains plug.  The peripheral components of the laser system should also be shut down and secured against  reactivation.

1 Attention: If the laser system has to be switched on for testing/measuring purposes, measures  for protection against laser radiation must be taken (use of protective goggles, setup and  positioning of partition walls, attachment of warning signs and barriers, etc.). These measures  must be coordinated with the laser protection officer.

The DIN EN 207 Filter und Augenschutzgeräte gegen Laserstrahlung (BS EN 207 Filters and Eye Protectors Against Laser Radiation (Laser Eye-Protectors)), DIN EN 60825-1 Sicherheit von  Lasereinrichtungen (BS EN 60825-1 Safety of Laser Products), and DIN EN 60825-4 Sicherheit  von Laserschutzwänden (BS EN 60825- 4 Safety of Laser Products: Laser Guards) standards  must be observed.

1. Dismantle the galvo head, set it

aside, and cover the entry openings

with tape (tape to be used:

(see Chapter 3) to prevent the inside

of the galvo head from getting dirty.

2. Open the cover of the laser head.

Figure 7.5 Dismantling the galvo head

3. Unscrew the shutter and lay it down

next to the laser head.

3 Note: Do not disconnect the plug

in connection (-X25) because the laser will

not be activated.

1 Attention: Position the shutter so

that damage (e. g. when opening the

shutter) cannot occur! The shutter

remains in operation!

1 Attention: The shutter will no

longer block the laser beam!

4. Secure the laser against activation.

Figure 7.6 Dismantle the shutter

1 Attention: Take measures for  protection against laser radiation  (see page 63)!

RSM PowerLine F - Repair Manual 63

Adjustment tasks

2

1

5. If applicable, remove the beam  expander (1) (Figure 7.7) including  the mount (2).

3 Note: The current setting of the  beam expander should be noted so that  the correct focal distance can be reset  after any false adjustment of the beam  expander.

Figure 7.7 Remove the beam expander  Figure 7.8 Mounting the transducer disk

6. Insert the mains plug of the laser  system and switch on the main  switch.

1 Attention: If the emergency stop  button/main switch of the laser system is  out of reach, a second person must  always be present who can press the  emergency stop button/main switch in  case of an emergency.

7. Record the laser power curve. 3 Note: The power curve serves to  set the corresponding power and as a  reference value to determine whether the  shutter, beam expander and galvo head  are causing a loss of power (cutting the  beam, defective optical components, etc.)

8. Across from the galvo flange, mount  a transducer disk (see Chapter 3) on  a surface that absorbs laser beams  using suitable equipment.

9. Activate the laser system and set  the power in the VLM program

(approx. 7 ... 10 W).

3 Note: Maximum output power  PWL 10 F: 10 W, PWL 20 F: 20 W.

64 RSM PowerLine F - Repair Manual

Adjustment tasks

45°  Figure 7.9 Mounting the cross wires

1

2

10. Enter the settings in the machine  configuration (see Section 4.2,

page 29), open the shutter, start the  VLM program (see Section 4.1,  page 27) and check the emission of  the main and alignment lasers on  the transducer disk.

11. Switch of the laser.

12. Insert the cross wires and their seat  as shown in Figure 7.9 into the

adjustment tube. This will ensure  that a vertical cross is displayed  during the adjustment.

3 Note: In the following steps,  make sure that the cross wires are always  in the same installation position.

1 Attention: Careful handling of the  adjusting aids must be guaranteed (no  deformation of the cross-wires, no  excessive laser power)! Damaged  adjusting aids lead to imprecise adjusting  results!

3 Note: Adjusting aids to be used:  see Chapter 3.

13. Screw the adjustment tube (1)  (Figure 7.10) onto the base plate (2).

Figure 7.10 Adjustment tube and base plate

RSM PowerLine F - Repair Manual 65

Adjustment tasks

Figure 7.11 Installation on the galvo flange

1 2 3 4

Figure 7.12 Checking the beam adjustment 2

1

2

Figure 7.13 Beam adjustment

14. Screw on the base plate of the  adjustment tube to the galvo flange.

15. Switch on the laser, start the VLM  program and check the image of the  laser on the transducer disk.

Figure 7.12 shows examples for  possible image:

– Adjustment OK (1)

– Emission too low (2)

– Emission too far to the left (3) – Emission too far to the right (4)

16. Adjust the beam by adjusting the  support block.

– Loosen the fastening screws (1)  (Figure 7.13).

– Adjust using the support screws  (2).

– Retighten the fastening screws  after the adjustment.

66 RSM PowerLine F - Repair Manual

Adjustment tasks

3

4

5

The following settings are possible:

(1) Moving lengthwise,

(2) Moving across,

(3) Twisting,

(4) Tilt,

(5) Height adjustment.

2

1

Figure 7.14 Setting options

Figure 7.15 Moving the support block

Figure 7.16 Mount the second set of cross wires

1 Attention: If tools are used to  move the support block (see Figure 7.15),  make sure that cables in the laser head  are not crushed or damaged!

17. Shut the laser off after the  adjustment.

18. Mount the second set of cross-wires  on the adjustment tube.

RSM PowerLine F - Repair Manual 67

Adjustment tasks

1 2

3 4

Figure 7.17 Checking the beam adjustment 2

1

Figure 7.18 Mount the beam expander 1 Figure 7.19 Setting the beam expander

19. Switch on the laser, start the VLM  program and check the image of the  laser on the transducer disk.

Figure 7.17 shows examples for  possible image:

– Adjustment OK (1)

– Emission too low (2)

– Emission too far to the left (3) – Emission too far to the right (4) 3 Note: The front cross-wires must  be turned by 45° for more exact control of  the laser beam.

1 Attention: Do not reach into the  laser beam when turning the cross wires!

20. Shut the laser off after the  adjustment.

21. If applicable, mount the beam  expander (1) (Figure 7.18) including  the mount (2).

22. Switch on the laser, Start the VLM  program and check the image of the  laser on the transducer disk (see  Figure 7.12 and Figure 7.17).

23. Move the beam expander after  loosening the three fastening screws  (1) (Figure 7.19) and use them to set  it (roundness of the laser beam,  center beam emission). Figure 7.20 shows examples for possible image.

1 Attention: Do not reach into the  laser beam when setting the beam  expander!

68 RSM PowerLine F - Repair Manual

Adjustment tasks

Adjustment OK

Emission too far to the upper right

24. Readjust the support block, if  necessary (see steps 15 to 19). 25. Compare the power to determine  whether the beam expander (if  applicable) causes a loss of power.  Clean the beam expander, if

necessary.

26. Shut the laser off after the  adjustment.

Figure 7.20 Beam adjustment beam expander 1

2

Figure 7.21 Mount the shutter

27. Mount the shutter (1) (Figure 7.21)  including the strip (2).

1 Attention: Do not crush or jam  the cables and lines inside the laser head  during installation!

28. Switch on the laser, Start the VLM  program and check the image of the  laser on the transducer disk (see  Figure 7.12 and Figure 7.17) and  adjust as needed.

29. Compare the power to determine  whether the shutter causes a loss of  power. Realign the shutter as

needed.

30. Shut the laser off after the  adjustment.

31. Remove the cross wires and their  seat from the adjustment tube.

RSM PowerLine F - Repair Manual 69

Adjustment tasks

Cross wires in the base plate

Cross wires in the adjustment

32. Switch on the laser, Start the VLM  program and check the image of the  laser on the transducer disk. Adjust  as needed.

33. Shut the laser off after the  adjustment.

34. Remove the cross wires and their  seat from the base plate.

35. Screw the adjustment tube to the  base plate of the galvo flange.

36. Switch on the laser, Start the VLM  program and check the image of the  laser on the transducer disk. Adjust  as needed.

Figure 7.22 Beam adjustment OK

3 Note: Repeat adjustments until  no more deviations can be determined in  the image after the installation position  changes.

3 Note: If power losses due to the  shutter are still present after multiple  adjustments, contact ROFIN-SINAR.

37. If no more deviations in the image  can be determined, the adjustment  is okay (see Figure 7.22).

38. Check the adjustment of the beam  expander; set the value read before  the adjustment procedure if

necessary.

39. Mount the galvo head.

40. Mount the cover of the laser head. 41. Commission the laser system. 42. Check the laser power behind the

galvo head (on the workpiece) using  the power measurement device and  perform a test marking procedure.  Enter the measurement values in  the logbook.

70 RSM PowerLine F - Repair Manual

Adjustment tasks

7.2.2 Focal point

3 Note: The following tasks only apply to laser systems with installed beam expanders.

1 Attention: Switch off the laser system and secure it against reactivation. Pull the mains plug.  The peripheral components of the laser system should also be shut down and secured against  reactivation.

1 Attention: If the laser system has to be switched on for testing/measuring purposes, measures  for protection against laser radiation must be taken (use of protective goggles, setup and  positioning of partition walls, attachment of warning signs and barriers, etc.). These measures  must be coordinated with the laser protection officer.

The DIN EN 207 Filter und Augenschutzgeräte gegen Laserstrahlung (BS EN 207 Filters and Eye Protectors Against Laser Radiation (Laser Eye-Protectors)), DIN EN 60825-1 Sicherheit von  Lasereinrichtungen (BS EN 60825-1 Safety of Laser Products), and DIN EN 60825-4 Sicherheit  von Laserschutzwänden (BS EN 60825- 4 Safety of Laser Products: Laser Guards) standards  must be observed.

1 Attention: To adjust the focal point, the distance between the galvo head and workpiece may  not be changed as the field size otherwise shifts!

1. Unscrew the shutter and lay it down

next to the laser head.

3 Note: Do not disconnect the plug

in connection (-X25) because the laser will

not be activated.

1 Attention: Position the shutter so

that damage (e. g. when opening the

shutter) cannot occur! The shutter

remains in operation!

1 Attention: The shutter will no

longer block the laser beam!

2. Secure the shutter against

Figure 7.23 Dismantle the shutter

activation.

1 Attention: Take measures for  protection against laser radiation  (see page 71)!

3. Insert the mains plug of the laser  system and switch on the main  switch.

1 Attention: If the emergency stop  button/main switch of the laser system is  out of reach, a second person must  always be present who can press the  emergency stop button/main switch in  case of an emergency.

RSM PowerLine F - Repair Manual 71

Adjustment tasks

1 2

3 4

Figure 7.24 Checking the focus setting 1 Figure 7.25 Setting the focal point

4. Lay material (an anodized metal  sheet, coated paper) under the  galvo head.

5. Switch on the laser system using the  key switch.

6. Open the shutter.

7. Let the program for the cross in the  middle of the field with the material  run according to the laser

parameters.

8. Switch of the laser.

9. Check the image of the cross. The  focal point is correctly set when both  laser lines in the X and Y direction  have the same dimensions at the  smallest possible width.

– Adjustment OK (1) (circular

cross-section)

– Too wide (2) (circular cross

section)

– Y dimension too wide (3) (cross section elliptical, X axis > Y-axis) – X dimension too wide (4) (cross section elliptical, X axis < Y-axis) 10. If necessary, change the focal point

by adjusting the adjusting ring (1)  (Figure 7.25) on the beam expander. 1 Attention: Do not reach into the  laser beam when turning the adjusting  ring!

3 Note: If the adjusting ring cannot  be adjusted, remove the beam expander  and loosen the locking screw (1)  (Figure 7.26, page 73).

11. Move material under the galvo head,  open the shutter, and let the

program run.

12. Check the image of the cross. 3 Note: Repeat Steps 6 through 12 until the focal point is correctly set.

13. Shut the laser off after the  adjustment.

72 RSM PowerLine F - Repair Manual

Adjustment tasks

1  Figure 7.26 Fixing the beam expander

1

2

Figure 7.27 Mount the shutter

7.2.3 Alignment laser1

14. Removing the beam expander. 15. Fix the position of the beam  expander using the locking screw (1)  (Figure 7.26).

16. Mount the beam expander 17. Setting the beam expander  (see page 68).

18. Mount the shutter (1) (Figure 7.27)  including the strip (2).

1 Attention: Do not crush or jam  the cables and lines inside the laser head  during installation!

19. Check the adjustment of the laser  (see page 69).

20. Compare the power after the galvo  head to determine whether the

beam expander (if applicable),

shutter or galvo head causes a loss  of power. Clean, repair or align the  beam expander, if necessary.

21. Shut the laser off after the  adjustment.

22. Mount the cover of the laser head. 23. Commission the laser system.

The alignment laser has the same beam emission as the main laser. No additional adjustment  tasks are necessary.

1. Installation depends on the respective laser type

RSM PowerLine F - Repair Manual 73

Adjustment tasks

Notes

74 RSM PowerLine F - Repair Manual

Repair work

8 Repair work

3 Note: The "Maintenance Schedule" chapter in the RSM PowerLine F user manual must also be  observed. The necessary aids are listed in Chapter 3 of this repair manual and in the "Spare Parts"  chapter of the RSM PowerLine F user manual.

8.1 Changing the optical components

1 Attention: When changing the optical components, make sure that the surfaces of the new  components are always free of grease and dust.

8.1.1 Galvo head

1 Attention: Switch off the laser system and secure it against reactivation. Pull the mains plug.  The peripheral components of the laser system should also be shut down and secured against  reactivation.

3 Note: The necessary spare parts of the galvo head can be ordered individually from ROFIN SINAR. To observe the field geometry/correction after the change of the galvo head and/or  focusing optics, we recommend that you order the components with a compensation file created  by ROFIN-SINAR.

1. Loosen and remove the connecting

1 Figure 8.1 Galvo head connecting plug

plug of the galvo head (1)  (Figure 8.1).

RSM PowerLine F - Repair Manual 75

Repair work

1  Figure 8.2 Dismantling the galvo head

1 Figure 8.3 Removing the distance ring

Figure 8.4 Covering the entry openings

2. Loosen the fastening screws (1)  (Figure 8.2).

3. Remove the galvo head.

4. Removing the ring seal (1)  (Figure 8.3).

5. Cover the entry openings with tape  (tape to be used: see Chapter 3) to  prevent the inside of the galvo head  from getting dirty.

76 RSM PowerLine F - Repair Manual